Energy Saving Series Injection Molding Machine

Technical Parameter:

| Model |  | SE90 |  |  | SE130 |  |  | SE160 |  |  | SE220 |  |  | SE280 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| INJECTION UNIT |  | A | B | C | A | B | C | A | B | C | A | B | C | A | B | C |
| Screw Diameter | mm | 32 | 36 | 40 | 36 | 40 | 45 | 40 | 45 | 50 | 45 | 50 | 55 | 55 | 60 | 65 |
| Screw L/D Ratio | L/D | 24.7 | 22 | 19.8 | 24.4 | 22 | 19.5 | 24.7 | 22 | 19.8 | 24.4 | 22 | 20 | 24 | 22 | 20.3 |
| Shot Size (Theoretical) | cm3 | 126 | 160 | 197 | 194 | 240 | 303 | 276 | 350 | 432 | 405 | 500 | 650 | 638 | 760 | 890 |
| Injection Weight (PS) | g | 115 | 146 | 179 | 177 | 218 | 276 | 251 | 318 | 393 | 368 | 455 | 550 | 580 | 690 | 810 |
| Injection Rate | g/s | 75 | 95 | 117 | 99 | 122 | 155 | 119 | 151 | 187 | 147 | 182 | 220 | 215 | 255 | 330 |
| Plasticizing Capacity | g/s | 14.2 | 18 | 22.2 | 18 | 22.2 | 28.1 | 19.8 | 25 | 30.9 | 24 | 29.6 | 35.8 | 31.8 | 37.8 | 44.3 |
| Injection Pressure | MPa | 234 | 185 | 150 | 222 | 180 | 142 | 227 | 180 | 145 | 222 | 180 | 148 | 214 | 180 | 153 |
| Screw Speed | rpm | 0---245 |  |  | 0---225 |  |  | 0—225 |  |  | 0---190 |  |  | 0---185 |  |  |
| Clamp Tonnage | KN | 900 |  |  | 1300 |  |  | 1600 |  |  | 2200 |  |  | 2800 |  |  |
| Toggle Stroke | mm | 320 |  |  | 350 |  |  | 450 |  |  | 500 |  |  | 550 |  |  |
| Space Between Tie Bars | mm | 360x360 |  |  | 410×410 |  |  | 455×450 |  |  | 515×510 |  |  | 580×580 |  |  |
| Max.Mold Height | mm | 330 |  |  | 430 |  |  | 500 |  |  | 520 |  |  | 580 |  |  |
| Min.Mold Height | mm | 110 |  |  | 130 |  |  | 150 |  |  | 165 |  |  | 200 |  |  |
| Ejector Stroke | mm | 90 |  |  | 110 |  |  | 110 |  |  | 160 |  |  | 160 |  |  |
| Ejector Tonnage | KN | 28 |  |  | 35 |  |  | 35 |  |  | 65 |  |  | 65 |  |  |
| Ejector Number | PC | 5 |  |  | 5 |  |  | 5 |  |  | 9 |  |  | 9 |  |  |
| Max. Pump Pressure | MPa | 16 |  |  | 16 |  |  | 16 |  |  | 160 |  |  | 16 |  |  |
| Pump Motor Power | Kw | 11 |  |  | 15 |  |  | 18.5 |  |  | 222 |  |  | 30 |  |  |
| Heater Power | Kw | 6.5 |  |  | 8.75 |  |  | 9.84 |  |  | 12.73 |  |  | 16.9 |  |  |
| Machine Dimension(LxWxH) | m | 4.2×1.2×1.6 |  |  | 4.6×1.3×1.7 |  |  | 5.2×1.3×1.8 |  |  | 5.7×1.5×1.9 |  |  | 6.2×1.5×2.0 |  |  |
| Machine Weight | T | 3.3 |  |  | 3.7 |  |  | 5.2 |  |  | 7.0 |  |  | 9.0 |  |  |
| Hopper Capacity | Kg | 25 |  |  | 25 |  |  | 25 |  |  | 50 |  |  | 50 |  |  |
| Oil Tank Capacity | L | 220 |  |  | 236 |  |  | 306 |  |  | 380 |  |  | 500 |  |  |

| Model |  | SE330 |  |  | SE380 |  |  | SE480 |  |  | SE580 |  |  | SE680 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| INJECTION UNIT |  | A | B | C | A | B | C | A | B | C | A | B | C | A | B | C |
| Screw Diameter | mm | 60 | 65 | 70 | 65 | 70 | 75 | 40 | 45 | 50 | 80 | 85 | 90 | 85 | 90 | 100 |
| Screw L/D Ratio | L/D | 21.6 | 20 | 18.5 | 23.7 | 22 | 20.5 | 24.7 | 22 | 19.8 | 23.4 | 22 | 20.8 | 23.3 | 22 | 19.8 |
| Shot Size (Theoretical) | cm3 | 894 | 1050 | 1217 | 1164 | 1350 | 1550 | 276 | 350 | 432 | 2217 | 2502 | 2805 | 2502 | 2805 | 3464 |
| Injection Weight (PS) | g | 805 | 945 | 1095 | 1062 | 1232 | 1414 | 251 | 318 | 393 | 2023 | 2283 | 2560 | 2283 | 2560 | 3161 |
| Injection Rate | g/s | 262.4 | 308 | 357.2 | 331.3 | 384.3 | 441.1 | 119 | 151 | 187 | 440.2 | 497 | 557.1 | 486.1 | 545 | 672.8 |
| Plasticizing Capacity | g/s | 34.1 | 40 | 46.4 | 40.8 | 47.4 | 54.4 | 19.8 | 25 | 30.9 | 57.9 | 65.4 | 73.3 | 64.3 | 72.1 | 72.8 |
| Injection Pressure | MPa | 211 | 180 | 155 | 203 | 175 | 152 | 227 | 180 | 145 | 197 | 175 | 156 | 196 | 175 | 142 |
| Screw Speed | rpm | 0---200 |  |  | 0---180 |  |  | 0—160 |  |  | 0---180 |  |  | 0---160 |  |  |
| Clamp Tonnage | KN | 3300 |  |  | 3800 |  |  | 4800 |  |  | 5800 |  |  | 6800 |  |  |
| Toggle Stroke | mm | 600 |  |  | 680 |  |  | 730 |  |  | 830 |  |  | 930 |  |  |
| Space Between Tie Bars | mm | 620×620 |  |  | 720×720 |  |  | 760×760 |  |  | 830×830 |  |  | 910×910 |  |  |
| Max.Mold Height | mm | 650 |  |  | 720 |  |  | 750 |  |  | 820 |  |  | 900 |  |  |
| Min.Mold Height | mm | 220 |  |  | 250 |  |  | 250 |  |  | 300 |  |  | 330 |  |  |
| Ejector Stroke | mm | 160 |  |  | 200 |  |  | 250 |  |  | 250 |  |  | 280 |  |  |
| Ejector Tonnage | KN | 65 |  |  | 110 |  |  | 125 |  |  | 125 |  |  | 200 |  |  |
| Ejector Number | PC | 9 |  |  | 17 |  |  | 17 |  |  | 17 |  |  | 21 |  |  |
| Max. Pump Pressure | MPa | 16 |  |  | 16 |  |  | 16 |  |  | 16 |  |  | 16 |  |  |
| Pump Motor Power | Kw | 30 |  |  | 37 |  |  | 45 |  |  | 55.5 |  |  | 67 |  |  |
| Heater Power | Kw | 18.5 |  |  | 23.15 |  |  | 29.2 |  |  | 32.65 |  |  | 36.45 |  |  |
| Machine Dimension(LxWxH) | m | 7.0×1.8×2.1 |  |  | 7.0×1.9×2.1 |  |  | 8.7×2.0×2.1 |  |  | 10.4×2.2×2.2 |  |  | 11×2.3×2.3 |  |  |
| Machine Weight | T | 13 |  |  | 19 |  |  | 22 |  |  | 31 |  |  | 37 |  |  |
| Hopper Capacity | Kg | 50 |  |  | 50 |  |  | 50 |  |  | 100 |  |  | 100 |  |  |
| Oil Tank Capacity | L | 620 |  |  | 1000 |  |  | 1250 |  |  | 1280 |  |  | 1280 |  |  |

Technical Parameter:

Technical Parameter:

| Model |  | SE780 |  |  | SE1000 |  |  | SE1250 |  |  | SE1500 |  |  | SE1800 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| INJECTION UNIT |  | A | B | C | A | B | C | A | B | C | A | B | C | A | B | C |
| Screw Diameter | mm | 90 | 100 | 110 | 100 | 110 | 120 | 110 | 120 | 130 | 120 | 130 | 140 | 130 | 140 | 150 |
| Screw L/D Ratio | L/D | 24.4 | 22 | 20 | 24.2 | 22 | 20.2 | 24 | 22 | 20.3 | 23.8 | 22 | 20.4 | 23.7 | 22 | 20.5 |
| Shot Size (Theoretical) | cm3 | 3080 | 3800 | 4600 | 3802 | 4600 | 5482 | 5300 | 6300 | 7402 | 6398 | 7500 | 8708 | 8623 | 10000 | 11480 |
| Injection Weight (PS) | g | 2809 | 3468 | 4198 | 3473 | 4199 | 5003 | 4841 | 5755 | 6760 | 5842 | 6848 | 7951 | 7868 | 9135 | 10475 |
| Injection Rate | g/s | 545.9 | 674 | 815.5 | 663.6 | 803 | 955.6 | 798.2 | 950 | 1114.9 | 875.9 | 1028 | 1192.2 | 1004.5 | 1165 | 1337.3 |
| Plasticizing Capacity | g/s | 69.5 | 85.5 | 103.8 | 83.6 | 101.2 | 120.4 | 103.9 | 123.7 | 145.1 | 115.9 | 136.1 | 157.8 | 132 | 153.1 | 175.7 |
| Injection Pressure | MPa | 210 | 170 | 140 | 206 | 170 | 143 | 202 | 170 | 145 | 200 | 170 | 147 | 186 | 160 | 139 |
| Screw Speed | rpm | 0---150 |  |  | 0---130 |  |  | 0—120 |  |  | 0---100 |  |  | 0—80 |  |  |
| Clamp Tonnage | KN | 7800 |  |  | 10000 |  |  | 12500 |  |  | 15000 |  |  | 18000 |  |  |
| Toggle Stroke | mm | 1000 |  |  | 1150 |  |  | 1150 |  |  | 1260 |  |  | 1520 |  |  |
| Space Between Tie Bars | mm | 985×985 |  |  | 1180×1180 |  |  | 1220×1100 |  |  | 1320×1200 |  |  | 1500×1480 |  |  |
| Max.Mold Height | mm | 980 |  |  | 1100 |  |  | 1100 |  |  | 1400 |  |  | 1500 |  |  |
| Min.Mold Height | mm | 350 |  |  | 400 |  |  | 500 |  |  | 600 |  |  | 650 |  |  |
| Ejector Stroke | mm | 320 |  |  | 320 |  |  | 360 |  |  | 360 |  |  | 400 |  |  |
| Ejector Tonnage | KN | 200 |  |  | 200 |  |  | 260 |  |  | 260 |  |  | 400 |  |  |
| Ejector Number | PC | 21 |  |  | 21 |  |  | 21 |  |  | 25 |  |  | 25 |  |  |
| Max. Pump Pressure | MPa | 16 |  |  | 16 |  |  | 16 |  |  | 16 |  |  | 16 |  |  |
| Pump Motor Power | Kw | 74 |  |  | 89 |  |  | 104 |  |  | 120 |  |  | 133 |  |  |
| Heater Power | Kw | 44.8 |  |  | 51.9 |  |  | 58.25 |  |  | 66.5 |  |  | 75.55 |  |  |
| Machine Dimension(LxWxH) | m | 11.5×2.4×2.3 |  |  | 12.0×2.7×2.6 |  |  | 12.6×2.8×2.7 |  |  | 14.2×2.9×2.8 |  |  | 15.5×3.1×3.6 |  |  |
| Machine Weight | T | 46 |  |  | 58 |  |  | 62 |  |  | 85 |  |  | 118 |  |  |
| Hopper Capacity | Kg | 100 |  |  | 100 |  |  | 100 |  |  | 400 |  |  | 400 |  |  |
| Oil Tank Capacity | L | 1800 |  |  | 2200 |  |  | 2500 |  |  | 2500 |  |  | 2900 |  |  |

Technical Parameter:

| Model |  | SE2200 |  |  |  |  | SE2800 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| INJECTION UNIT |  | A | B | C |  | D | A | B |  |  |
| Screw Diameter | mm | 150 | 160 | 170 |  | 185 | 210 | 220 |  |  |
| Screw L/D Ratio | L/D | 23.5 | 22 | 20.7 |  | 22 | 22 | 22 |  |  |
| Shot Size (Theoretical) | cm3 | 13183 | 15000 | 16933 |  | 24192 | 41200 | 45236 |  |  |
| Injection Weight (PS) | g | 12043 | 13700 | 15468 |  | 22075 | 37600 | 41278 |  |  |
| Injection Rate | g/s | 1335 | 1519 | 1714.8 |  | 1700 | 2000 | 2200 |  |  |
| Plasticizing Capacity | g/s | 158.3 | 180.1 | 203.3 |  | 223 | 280 | 300 |  |  |
| Injection Pressure | MPa | 170 | 150 | 133 |  | 140 | 155 | 141 |  |  |
| Screw Speed | rpm | 0---80 |  | 0---60 |  |  | 0—60 |  | 10---60 |  |
| Clamp Tonnage | KN | 22000 |  |  |  |  | 28000 |  |  |  |
| Toggle Stroke | mm | 1750 |  |  |  |  | 1850 |  |  |  |
| Space Between Tie Bars | mm | 1650×1620 |  |  |  |  | 1820×1620 |  |  |  |
| Max.Mold Height | mm | 1600 |  |  |  |  | 1600 |  |  |  |
| Min.Mold Height | mm | 700 |  |  |  |  | 700 |  |  |  |
| Ejector Stroke | mm | 400 |  |  |  |  | 400 |  |  |  |
| Ejector Tonnage | KN | 500 |  |  |  |  | 700 |  |  |  |
| Ejector Number | PC | 29 |  |  |  |  | 29 |  |  |  |
| Max. Pump Pressure | MPa | 16 |  |  |  |  | 16 |  |  |  |
| Pump Motor Power | Kw | 153.5 |  |  |  |  | 220 |  |  |  |
| Heater Power | Kw | 98.7 |  |  | 133.7 |  | 177 |  |  | 185.45 |
| Machine Dimension(LxWxH) | m | 17.5×4.2×3.7 |  |  | 18.5×4.8×4.3 |  | 18.5×3.7×3.6 |  |  |  |
| Machine Weight | T | 136 |  |  | 140 |  | 195 |  |  |  |
| Hopper Capacity | Kg | 400 |  |  |  |  |  |  |  |  |
| Oil Tank Capacity | L | 4100 |  |  |  |  |  |  |  |  |

List of hydraulic elements , electric elements of SE90-SE220

| NO | NAME | BRAND |
| --- | --- | --- |
| 1 | COMPUTER | TACHMATION |
| 2 | SERVO CONTROLLER | BOSCH-REXROTH |
| 3 | GEAR PUMP | BOSCH-REXROTH |
| 4 | SERVO MOTOR | BOSCH-REXROTH |
| 5 | DIRECTIONAL VALVE | BOSCH-REXROTH |
| 6 | HYFRAULIC MOTOR | INTERMOT OR DINAMIC |
| 7 | AIR BREAKER | CHNT |
| 8 | CONTACTOR | SCHNEIDER |
| 9 | THERMAL OVERLOAD RELAY | SCHNEIDER |
| 10 | LINEAR TRANSDUCER | VISHAY |
| 11 | LOCATION LIMIT SWITCH | NAIS |
| 13 | COMPACT SEALS | BUSAK-SHAMBAN |

Specialty:

1. High precision control

2. Quick response

3. High sensitivity

4. Running in low noise

5. Energy economical and environmental protection

6. Energy saving can reach 20%-80%

FTN Series Injection Machine

Technical Parameter:

| Model |  | FT90 |  |  | FT130 |  |  | FT160 |  |  | FT220 |  |  | FT330 |  |  | FT380 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| INJECTION UNIT |  | A | B | C | A | B | C | A | B | C | A | B | C | A | B | C | A | B | C |
| Screw Diameter | mm | 32 | 36 | 40 | 36 | 40 | 45 | 40 | 45 | 50 | 45 | 50 | 55 | 60 | 65 | 70 | 65 | 70 | 75 |
| Screw L/D Ratio | L/D | 24.7 | 22 | 19.8 | 24.4 | 22 | 19.5 | 24.7 | 22 | 19.8 | 24.4 | 22 | 20 | 21.6 | 20 | 18.5 | 23.7 | 22 | 20.5 |
| Shot Size (Theoretical) | cm3 | 126 | 160 | 197 | 194 | 240 | 303 | 276 | 350 | 432 | 405 | 500 | 650 | 894 | 1050 | 1217 | 1164 | 1350 | 1550 |
| Injection Weight (PS) | g | 115 | 146 | 179 | 177 | 218 | 276 | 251 | 318 | 393 | 368 | 455 | 550 | 805 | 945 | 1095 | 1062 | 1232 | 1414 |
| Injection Rate | g/s | 75 | 95 | 117 | 99 | 122 | 155 | 119 | 151 | 187 | 147 | 182 | 220 | 262.4 | 308 | 357.2 | 331.3 | 384.3 | 441.1 |
| Plasticizing Capacity | g/s | 14.2 | 18 | 22.2 | 18 | 22.2 | 28.1 | 19.8 | 25 | 30.9 | 24 | 29.6 | 35.8 | 34.1 | 40 | 46.4 | 40.8 | 47.4 | 54.4 |
| Injection Pressure | MPa | 234 | 185 | 150 | 222 | 180 | 142 | 227 | 180 | 145 | 222 | 180 | 148 | 211 | 180 | 155 | 203 | 175 | 152 |
| Screw Speed | rpm | 0-245 |  |  | 0---225 |  |  | 0—225 |  |  | 0---190 |  |  | 0---200 |  |  | 0---180 |  |  |
| Clamp Tonnage | KN | 900 |  |  | 1300 |  |  | 1600 |  |  | 2200 |  |  | 3300 |  |  | 3800 |  |  |
| Toggle Stroke | mm | 320 |  |  | 350 |  |  | 450 |  |  | 500 |  |  | 600 |  |  | 680 |  |  |
| Space Between Tie Bars | mm | 360*360 |  |  | 410×410 |  |  | 455×450 |  |  | 515×510 |  |  | 620×620 |  |  | 720×720 |  |  |
| Max.Mold Height | mm | 330 |  |  | 430 |  |  | 500 |  |  | 520 |  |  | 650 |  |  | 720 |  |  |
| Min.Mold Height | mm | 110 |  |  | 130 |  |  | 150 |  |  | 165 |  |  | 220 |  |  | 250 |  |  |
| Ejector Stroke | mm | 90 |  |  | 110 |  |  | 110 |  |  | 160 |  |  | 160 |  |  | 200 |  |  |
| Ejector Tonnage | KN | 65 |  |  | 35 |  |  | 35 |  |  | 65 |  |  | 65 |  |  | 110 |  |  |
| Ejector Number | PC | 28 |  |  | 5 |  |  | 5 |  |  | 9 |  |  | 9 |  |  | 17 |  |  |
| Max. Pump Pressure | MPa | 16 |  |  | 16 |  |  | 16 |  |  | 160 |  |  | 16 |  |  | 16 |  |  |
| Pump Motor Power | Kw | 11 |  |  | 15 |  |  | 18.5 |  |  | 222 |  |  | 30 |  |  | 37 |  |  |
| Heater Power | Kw | 6.5 |  |  | 8.75 |  |  | 9.84 |  |  | 12.73 |  |  | 18.5 |  |  | 23.15 |  |  |
| Machine Dimension(LxWxH) | m | 4.2 x 1.2 x 1.6 |  |  | 4.6×1.3×1.7 |  |  | 5.2×1.3×1.8 |  |  | 5.7×1.5×1.9 |  |  | 7.0×1.8×2.1 |  |  | 7.0×1.9×2.1 |  |  |
| Machine Weight | t | 3.3 |  |  | 3.7 |  |  | 5.2 |  |  | 7.0 |  |  | 13 |  |  | 19 |  |  |
| Hopper Capacity | Kg | 25 |  |  | 25 |  |  | 25 |  |  | 50 |  |  | 50 |  |  | 50 |  |  |
| Oil Tank Capacity | L | 220 |  |  | 236 |  |  | 306 |  |  | 380 |  |  | 620 |  |  | 1000 |  |  |

Technical Parameter:

| Model |  | FT480 |  |  | FT580 |  |  | FT680 |  |  | FT780 |  |  | FT1000 |  |  | FT1250 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| INJECTION UNIT |  | A | B | C | A | B | C | A | B | C | A | B | C | A | B | C | A | B | C |
| Screw Diameter | mm | 75 | 80 | 85 | 80 | 85 | 90 | 85 | 90 | 100 | 90 | 100 | 110 | 100 | 110 | 120 | 110 | 120 | 130 |
| Screw L/D Ratio | L/D | 23.5 | 22 | 20.7 | 23.4 | 22 | 20.8 | 23.3 | 22 | 19.8 | 24.4 | 22 | 20 | 24.2 | 22 | 20.2 | 24 | 22 | 20.3 |
| Shot Size (Theoretical) | cm3 | 1758 | 2000 | 2258 | 2217 | 2502 | 2805 | 2502 | 2805 | 3464 | 3080 | 3800 | 4600 | 3802 | 4600 | 5482 | 5300 | 6300 | 7402 |
| Injection Weight (PS) | g | 1605 | 1825 | 2060 | 2023 | 2283 | 2560 | 2283 | 2560 | 3161 | 2809 | 3468 | 4198 | 3473 | 4199 | 5003 | 4841 | 5755 | 6760 |
| Injection Rate | g/s | 373.5 | 425 | 480 | 440.2 | 497 | 557.1 | 486.1 | 545 | 672.8 | 545.9 | 674 | 815.5 | 663.6 | 803 | 955.6 | 798.2 | 950 | 1114.9 |
| Plasticizing Capacity | g/s | 53 | 60.3 | 68.1 | 57.9 | 65.4 | 73.3 | 64.3 | 72.1 | 72.8 | 69.5 | 85.5 | 103.8 | 83.6 | 101.2 | 120.4 | 103.9 | 123.7 | 145.1 |
| Injection Pressure | MPa | 199 | 175 | 155 | 197 | 175 | 156 | 196 | 175 | 142 | 210 | 170 | 140 | 206 | 170 | 143 | 202 | 170 | 145 |
| Screw Speed | rpm | 0—160 |  |  | 0---180 |  |  | 0---160 |  |  | 0---150 |  |  | 0---130 |  |  | 0—120 |  |  |
| Clamp Tonnage | KN | 4800 |  |  | 5800 |  |  | 6800 |  |  | 7800 |  |  | 10000 |  |  | 12500 |  |  |
| Toggle Stroke | mm | 730 |  |  | 830 |  |  | 930 |  |  | 1000 |  |  | 1150 |  |  | 1150 |  |  |
| Space Between Tie Bars | mm | 760×760 |  |  | 830×830 |  |  | 910×910 |  |  | 985×985 |  |  | 1180×1180 |  |  | 1220×1100 |  |  |
| Max.Mold Height | mm | 750 |  |  | 820 |  |  | 900 |  |  | 980 |  |  | 1100 |  |  | 1100 |  |  |
| Min.Mold Height | mm | 250 |  |  | 300 |  |  | 330 |  |  | 350 |  |  | 400 |  |  | 500 |  |  |
| Ejector Stroke | mm | 250 |  |  | 250 |  |  | 280 |  |  | 320 |  |  | 320 |  |  | 360 |  |  |
| Ejector Tonnage | KN | 125 |  |  | 125 |  |  | 200 |  |  | 200 |  |  | 200 |  |  | 260 |  |  |
| Ejector Number | PC | 17 |  |  | 17 |  |  | 21 |  |  | 21 |  |  | 21 |  |  | 21 |  |  |
| Max. Pump Pressure | MPa | 16 |  |  | 16 |  |  | 16 |  |  | 16 |  |  | 16 |  |  | 16 |  |  |
| Pump Motor Power | Kw | 45 |  |  | 55.5 |  |  | 67 |  |  | 74 |  |  | 89 |  |  | 104 |  |  |
| Heater Power | Kw | 29.2 |  |  | 32.65 |  |  | 36.45 |  |  | 44.8 |  |  | 51.9 |  |  | 58.25 |  |  |
| Machine Dimension(LxWxH) | m | 8.7×2.0×2.1 |  |  | 10.4×2.2×2.2 |  |  | 11×2.3×2.3 |  |  | 11.5×2.4×2.3 |  |  | 12.0×2.7×2.6 |  |  | 12.6×2.8×2.7 |  |  |
| Machine Weight | t | 22 |  |  | 31 |  |  | 37 |  |  | 46 |  |  | 58 |  |  | 62 |  |  |
| Hopper Capacity | Kg | 50 |  |  | 100 |  |  | 100 |  |  | 100 |  |  | 100 |  |  | 100 |  |  |
| Oil Tank Capacity | L | 1250 |  |  | 1280 |  |  | 1280 |  |  | 1800 |  |  | 2200 |  |  | 2500 |  |  |

Technical Parameter:

| Model |  | FT1500 |  |  | FT1800 |  |  | FT2200 |  |  |  | FT2800 |  | FT3600 | FT4000 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| INJECTION UNIT |  | A | B | C | A | B | C | A | B | C | D | A | B |  |  |
| Screw Diameter | mm | 120 | 130 | 140 | 130 | 140 | 150 | 150 | 160 | 170 | 185 | 210 | 220 | 230 | 240 |
| Screw L/D Ratio | L/D | 23.8 | 22 | 20.4 | 23.7 | 22 | 20.5 | 23.5 | 22 | 20.7 | 22 | 22 | 22 | 22 | 22 |
| Shot Size (Theoretical) | cm3 | 6398 | 7500 | 8708 | 8623 | 10000 | 11480 | 13183 | 15000 | 16933 | 24192 | 41200 | 45236 | 50270 | 56600 |
| Injection Weight (PS) | g | 5842 | 6848 | 7951 | 7868 | 9135 | 10475 | 12043 | 13700 | 15468 | 22075 | 37600 | 41278 | 45870 | 51500 |
| Injection Rate | g/s | 875.9 | 1028 | 1192.2 | 1004.5 | 1165 | 1337.3 | 1335 | 1519 | 1714.8 | 1700 | 2000 | 2200 | 2400 | 2750 |
| Plasticizing Capacity | g/s | 115.9 | 136.1 | 157.8 | 132 | 153.1 | 175.7 | 158.3 | 180.1 | 203.3 | 223 | 280 | 300 | 320 | 370 |
| Injection Pressure | MPa | 200 | 170 | 147 | 186 | 160 | 139 | 170 | 150 | 133 | 140 | 155 | 141 | 160 | 160 |
| Screw Speed | rpm | 0-100 |  |  | 0-80 |  |  | 0-80 |  | 0-60 |  | 0-60 | 10-60 |  | 0—45 |
| Clamp Tonnage | KN | 15000 |  |  | 18000 |  |  | 22000 |  |  |  | 28000 |  | 36000 | 40000 |
| Toggle Stroke | mm | 1260 |  |  | 1520 |  |  | 1750 |  |  |  | 1850 |  | 2250 | 2350 |
| Space Between Tie Bars | mm | 1320×1200 |  |  | 1500×1480 |  |  | 1650×1620 |  |  |  | 1820×1620 |  | 2160×1900 | 2400×2200 |
| Max.Mold Height | mm | 1400 |  |  | 1500 |  |  | 1600 |  |  |  | 1600 |  | 2100 | 2250 |
| Min.Mold Height | mm | 600 |  |  | 650 |  |  | 700 |  |  |  | 700 |  | 800 | 1000 |
| Ejector Stroke | mm | 360 |  |  | 400 |  |  | 400 |  |  |  | 400 |  | 550 | 600 |
| Ejector Tonnage | KN | 260 |  |  | 400 |  |  | 500 |  |  |  | 700 |  | 700 | 700 |
| Ejector Number | PC | 25 |  |  | 25 |  |  | 29 |  |  |  | 29 |  | 25 | 25 |
| Max. Pump Pressure | MPa | 16 |  |  | 16 |  |  | 16 |  | 16 |  | 16 |  | 16 | 16 |
| Pump Motor Power | Kw | 120 |  |  | 133 |  |  | 153.5 |  | 153.5 |  | 220 | 200 | 275 | 330 |
| Heater Power | Kw | 66.5 |  |  | 75.55 |  |  | 98.7 |  | 133.7 |  | 177 | 185.45 | 202 | 250 |
| Machine Dimension(LxWxH) | m | 14.2×2.9×2.8 |  |  | 15.5×3.1×3.6 |  |  | 17.5×4.2×3.7 |  | 18.5×4.8×4.3 |  | 18.5×3.7×3.6 |  | 21.5×5.1×3.8 | 24.5×5.5×4.5 |
| Machine Weight | t | 85 |  |  | 118 |  |  | 136 |  | 140 |  | 195 |  | 280 | 320 |
| Hopper Capacity | Kg | 400 |  |  | 400 |  |  | 400 |  |  |  | 400 |  | 400 | 400 |
| Oil Tank Capacity | L | 2500 |  |  | 2900 |  |  | 4100 |  |  |  | 5500 |  | 7000 | 7000 |

Configuration ：

(The specifications are subject to change without notice )

List of hydraulic elements , electric elements of FTN90－FTN280

| No | NAME | BRAND |
| --- | --- | --- |
| 1 | Computer | TECHMATION |
| 2 | Proportional Flow Control and Relief Valves | YUKEN |
| 3 | Directional Valve | Rexroth |
| 4 | Vane Pump | VICKERS or DENISON |
| 5 | Hydraulic Motor | Intermot or Dinamic |
| 6 | Air Breaker | CHNT |
| 7 | Contactor | SCHNEIDER |
| 8 | Thermal Overload Relay | SCHNEIDER |
| 9 | Linear Transducer | VISHAY |
| 10 | Location Limit Switch | NAiS |
| 11 | Seals | BUSAK-SHAMBAN |
| 12 | Electromotor | Beijing |

Application area：

Daily use, Electronics, Cars, Logistics, Medical treatment, Engineering , Channel and so on

FE Series Injection Machine :  FE-160

Specialty:

1. High precision control

2. High speed

3. Stable

4. Energy economical and environmental protection

Production:

Eyeglass, Mobilephone casing, Electrical shell